ENVIRONMENTAL AND SOCIAL MANAGEMENT PLAN

(ESMP)

for the

Perlepnica Dam – Priority Dam Safety Interventions

Prepared under

Fostering and Leveraging Opportunities for Water Security –FLOWS

# June 2026

TABLE OF CONTENTS

EXECUTIVE SUMMARY

- INTRODUCTION

1.1 Background

1.2 Purpose of the ESMP

1.3 Scope of the ESMP

- LEGAL, POLICY AND INSTITUTIONAL FRAMEWORK

2.1 Applicable National Legislation

2.2 World Bank Environmental and Social Standards

- SUBPROJECTENVIRONMENTAL AND SOCIAL BASELINE CONDITIONS

3.1 Subproject Setting

3.2 Physical Environment

3.3 Water Resources

3.4 Biological Environment

3.5 Socio-Economic Environment

3.6 Existing Dam Safety Conditions

- ENVIRONMENTAL AND SOCIAL RISKS AND IMPACTS

4.1 Overview

4.2 Environmental Risks and Impacts

4.2.1 Dust Emissions

4.2.2 Noise and Vibration

4.2.3 Water Quality Risks

4.2.4 Construction Waste Generation

4.2.5 Hazardous Materials and Associated Waste

4.3 Social Risks and Impacts

4.3.1 Occupational Health and Safety Risks

4.3.2 Community Health and Safety Risks

4.3.3 Labour and Working Conditions

4.4 Dam Safety Related Risks

- ENVIRONMENTAL AND SOCIAL MITIGATION MEASURES

5.1 General

5.2 Environmental and Social Mitigation Measures

- ENVIRONMENTAL AND SOCIAL MONITORING PLAN

6.1 Monitoring Objectives

6.2 Environmental and Social Monitoring Plan

- INSTITUTIONAL ARRANGEMENTS, CAPACITY BUILDING AND TRAINING

7.1 Institutional Arrangements

7.2 Institutional Responsibilities

7.3 Capacity Building and Training

7.4 Reporting Arrangements

- GRIEVANCE MECHANISM

- IMPLEMENTATION SCHEDULE AND INTEGRATION OF ESMP INTO SUBPROJECT IMPLEMENTATION

9.1 ESMP Implementation Schedule

9.2 Integration of ESMP into Subproject Implementation

List of Tables

- Table 1. Summary Sheet

- Table 2-1. Applicable National Legislation

- Table 2-2. Applicability of World Bank Environmental and Social Standards

- Table 5-1. Environmental and Social Mitigation Measures

- Table 6-1. Environmental and Social Monitoring Plan

- Table 7-1. Institutional Responsibilities

- Table 7-2. Training Requirements

- Table 9-1. ESMP Implementation Schedule

Table 1. Summary Sheet

| Item | Description |
| --- | --- |
| Name of the Subproject | Perlepnica Dam – Priority Dam Safety Interventions |
| SubsubprojectSpecification | Upgrade of the seepages monitoring system; Concrete restoration works, including: Repair cracks along the spillway sill and along spillway chute wall; Repair cracks in the bottom outlet gallery to minimize the leakages with injection; Rehabilitate the terminal part of the chute and plunge pool; EM and HSS maintenance works, including: Replace the hydromechanical components (valves and pipes) of the bottom outlet and intake; Completing the installation of monitoring instruments. |
| Subproject Location | Perlepnica Dam, Municipality of Gjilan, Republic of Kosovo |
| Number of Beneficiaries | Beneficiaries include communities, public utilities and other water users dependent on water resources and services associated with the Perlepnica Reservoir. |
| Sector and Type of Subproject | Water Sector – Dam Safety Rehabilitation and Water Supply Infrastructure |
| Client | Ministry of Environment and Spatial Planning (MESP), through the Project Management Team (PMT) |
| Beneficiary | KRU Hidromorava |
| Implementation Modality | Implemented through a competitively selected Contractor under the supervision of a Supervising Engineer |
| Size of the Subproject | Rehabilitation works within the existing Perlepnica Dam footprint and associated operational facilities |
| Field Visit (Yes/No; Include Date) | Yes – Site visits undertaken during dam safety assessments and detailed design preparation |
| Subproject Risk Category (Low to High) | Moderate |
| Prepared by: | FLOWS PMT |

## EXECUTIVE SUMMARY

Perlepnica Dam is an important water supply infrastructure located in the Municipality of Gjilan and serves as a key source of drinking water for communities supplied through the service area of the Regional Water Company Hidromorava (KRU Hidromorava).

To support its continued safe operation and long-term reliability, dam safety assessments and detailed engineering design studies were undertaken between 2024 and 2026 under the FLOWS Project through specialized engineering consultants. The findings and recommendations of these studies formed the basis for identification of priority rehabilitation measures to be implemented under the subproject. This ESMP has been prepared by the Project Management Team (PMT) based on the findings of these studies and the associated engineering designs prepared under the FLOWS Project.

At this phase, the proposed Perlepnica Dam rehabilitation works include the following priority dam safety interventions:

- Upgrade of the seepage monitoring system;

- Concrete restoration works, including:
  - Repair of cracks along the spillway sill and spillway chute walls;
  - Repair of cracks in the bottom outlet gallery to reduce leakages through injection works;
  - Rehabilitation of the terminal section of the spillway chute and plunge pool;

- Electromechanical (EM) and Hydro-Mechanical Safety System (HSS) maintenance works, including:
  - Replacement of hydromechanical components (pipes, valves and associated fittings) of the bottom outlet and intake systems;

- Completion, testing and commissioning of monitoring instruments and associated monitoring systems.

Accordingly, this ESMP has been prepared for the priority rehabilitation measures included in the current Subproject scope.

This Environmental and Social Management Plan (ESMP) defines the environmental and social mitigation, monitoring, institutional, training and reporting measures required during implementation of the Project. The ESMP has been prepared in accordance with the World Bank Environmental and Social Framework (ESF), relevant Environmental and Social Standards (ESSs), and applicable legislation of Kosovo.

Environmental and social impacts associated with the proposed rehabilitation works are expected to be site-specific, temporary and manageable through implementation of appropriate mitigation measures. The principal environmental risks include generation of construction and demolition waste, dust and noise emissions, temporary disturbance associated with repair works, handling of construction chemicals and grouts used for crack injection, management of deteriorated pipes and valves removed from service, accidental contamination of reservoir waters, and generation of hazardous and electronic wastes. Social risks are primarily associated with occupational health and safety, work at height, confined-space entry within galleries and intake structures, electrical and mechanical safety risks, and community health and safety during construction activities.

Implementation of the proposed interventions is expected to provide long-term benefits through improved dam safety, enhanced seepage monitoring capability, rehabilitation of deteriorated spillway and outlet structures, improved reliability of hydraulic and monitoring systems, and reduced risks associated with seepage, leakage and deterioration of critical dam infrastructure.

The Subproject will contribute to the continued safe operation of Perlepnica Dam and the reliability of water supply services provided to communities within the KRU Hidromorava service area.

# 1. INTRODUCTION

## 1.1 Background

Perlepnica Dam is an important water supply infrastructure located in the Municipality of Gjilan and represents a significant source of drinking water for communities served by the Regional Water Company Hidromorava. The reservoir plays an important role in supporting water security and the reliable supply of potable water within the KRU Hidromorava service area.

As part of the FLOWS Project, dam safety assessments and detailed engineering design studies were undertaken through specialized consultants to evaluate the condition of Perlepnica Dam and its associated infrastructure. The assessment identified deficiencies affecting seepage monitoring systems, spillway structures, hydraulic infrastructure, monitoring instrumentation and electromechanical components. Although the overall structural integrity of the dam was found to be satisfactory, several maintenance-related deficiencies and operational issues were identified, requiring rehabilitation to improve dam safety, monitoring capability and operational reliability.

Based on the recommendations of the dam safety assessment and the approved priority rehabilitation program, a number of rehabilitation interventions have been selected for implementation under the Project.

## 1.2 Purpose of the ESMP

The purpose of this Environmental and Social Management Plan (ESMP) is to define the mitigation, monitoring, institutional and reporting measures required to avoid, minimize, mitigate and manage environmental and social risks and impacts associated with implementation of the proposed rehabilitation works.

The ESMP has been prepared in accordance with the requirements of the World Bank Environmental and Social Framework (ESF) and forms an integral part of Subproject implementation.

## 1.3 Scope of the ESMP

This ESMP applies exclusively to the rehabilitation activities financed under the Project.

The scope of the ESMP includes the following activities:

Activity 1 – Upgrade of the Seepage Monitoring System

The works include: rehabilitation and upgrading of seepage monitoring facilities; repair or replacement of existing seepage measurement structures; completion of monitoring installations; connection to the data acquisition system; testing, calibration and commissioning of monitoring equipment.

Activity 2 – Concrete Restoration Works

The works include: repair of cracks along the spillway sill and spillway chute walls; concrete repairs; reinforcement treatment; crack sealing; injection works; surface restoration; and application of protective repair systems where required.

Activity 3 – Repair of Cracks in the Bottom Outlet Gallery

The works include: cleaning and preparation of damaged surfaces; crack injection and sealing works; repair of leakage points; concrete restoration; and associated waterproofing measures within the gallery.

Activity 4 – Rehabilitation of the Terminal Section of the Spillway Chute and Plunge Pool

The works include: rehabilitation of eroded sections of the spillway discharge area; repair and restoration of damaged concrete and structural elements; stabilization and protection works within the plunge pool area; and associated civil works required to restore hydraulic performance and erosion protection.

Activity 5 – Removal and Replacement of Hydraulic Components

The works include: removal of existing pipes, valves and associated hydromechanical equipment; supply and installation of new hydraulic components; associated mechanical works; testing; and commissioning of the rehabilitated systems.

Activity 6 – Completion of Monitoring Instrument Installation

The works include: completion of installation of monitoring instruments; cabling and communication connections; functional testing and calibration; integration with the data acquisition system; commissioning; and operator training.

## 2. LEGAL, POLICY AND INSTITUTIONAL FRAMEWORK

### 2.1 Applicable National Legislation

The Subproject shall be implemented in accordance with the applicable environmental, occupational health and safety, water management, labour and construction legislation of the Republic of Kosovo, as well as the requirements of the World Bank Environmental and Social Framework (ESF). Compliance with applicable legal requirements shall be ensured throughout Subproject implementation.

The principal legal instruments relevant to the proposed rehabilitation works are summarized below.

Table 2-1. Applicable National Legislation

| Legal Instrument | Relevance to the Project |
| --- | --- |
| Law No. 03/L-025 on Environmental Protection | Establishes the general principles of environmental protection, pollution prevention and sustainable development and provides the overall framework for management of environmental impacts associated with the rehabilitation works. |
| Law No. 04/L-147 on Waters of Kosovo | Regulates the protection, management and sustainable use of water resources. The law is particularly relevant as the rehabilitation works will be undertaken within an operational drinking water reservoir and associated hydraulic infrastructure. |
| Law No. 04/L-060 on Waste | Establishes requirements for collection, segregation, transport, recovery and disposal of waste. The Subprojectwill generate construction waste, scrap metal, packaging waste and limited quantities of hazardous waste during rehabilitation activities. |
| Law No. 04/L-161 on Safety and Health at Work | Establishes requirements for occupational health and safety, accident prevention and worker protection. The law is relevant due to work at height, confined-space entry, electrical works and handling of heavy equipment. |
| Law No. 03/L-212 on Labour | Regulates employment relationships, workers’ rights, working conditions and employer obligations. The law applies to all Subproject workers engaged during implementation. |
| Law No. 04/L-110 on Construction | Establishes technical, administrative and safety requirements governing construction activities and applies to civil, mechanical and electrical rehabilitation works. |
| Law No. 04/L-197 on Chemicals | Regulates the handling, storage, transport and disposal of chemicals and hazardous substances. The law is relevant due to the use of coatings, resins, corrosion protection products and other chemical materials during rehabilitation works. |

The proposed activities consist exclusively of rehabilitation and upgrading works within the existing operational footprint of Perlepnica Dam and its associated infrastructure.

The Subproject does not involve expansion of the reservoir, modification of the dam embankment, land acquisition, involuntary resettlement, physical displacement or restrictions on land use.

Environmental and social impacts are expected to be localized, temporary and manageable through implementation of good construction practices and the mitigation and monitoring measures defined in this ESMP.

### 2.2 World Bank Environmental and Social Standards

The Subprojectis financed by the World Bank and is subject to the requirements of the Environmental and Social Framework (ESF). The applicability of the Environmental and Social Standards (ESSs) was assessed based on the nature and scope of the proposed rehabilitation works.

Table 2-2. Applicability of World Bank Environmental and Social Standards

| ESS | Applicability | Justification |
| --- | --- | --- |
| ESS1 – Assessment and Management of Environmental and Social Risks and Impacts | Applicable | Rehabilitation activities may generate environmental and social risks requiring assessment, mitigation, monitoring and management. This ESMP serves as the primary environmental and social management instrument for the Project. The Subproject includes rehabilitation of spillway structures, seepage monitoring systems, hydraulic infrastructure and monitoring instrumentation. |
| ESS2 – Labor and Working Conditions | Applicable | The Subproject will involve direct and contracted workers engaged in civil, mechanical and instrumentation works. Occupational health and safety risks include work at height, confined-space entry, excavation works, handling of heavy equipment and mechanical installations. |
| ESS3 – Resource Efficiency and Pollution Prevention and Management | Applicable | The Subproject will generate construction waste, scrap metal, deteriorated pipes and valves, packaging waste and limited hazardous waste. The rehabilitation works will also involve the use of grouts, sealants, coatings, lubricants and other materials requiring appropriate pollution prevention and waste management measures. |
| ESS4 – Community Health and Safety | Applicable | Rehabilitation activities will be undertaken within an operational dam and water supply facility. Measures will be required to protect workers, visitors, dam operators and surrounding communities during implementation of the works. |
| ESS5 – Land Acquisition, Restrictions on Land Use and Involuntary Resettlement | Not Applicable | The rehabilitation works will be implemented entirely within the existing operational footprint of Perlepnica Dam and associated infrastructure. No land acquisition, physical displacement, economic displacement or restrictions on land use are required. |
| ESS6 – Biodiversity Conservation and Sustainable Management of Living Natural Resources | Not Applicable | The works are limited to rehabilitation of existing dam infrastructure and will be implemented within previously disturbed operational areas. No impacts on natural habitats, protected areas or biodiversity are anticipated. |
| ESS7 – Indigenous Peoples/Sub-Saharan African Historically Underserved Traditional Local Communities | Not Applicable | No groups meeting the requirements of ESS7 are present within the Subproject area. |
| ESS8 – Cultural Heritage | Not Applicable* | No known cultural heritage resources are located within the Subproject footprint. A Chance Finds Procedure shall nevertheless be implemented during construction. |
| ESS9 – Financial Intermediaries | Not Applicable | The Subproject does not involve financial intermediaries. |
| ESS10 – Stakeholder Engagement and Information Disclosure | Applicable | Prior to commencement of works, the PMT, in coordination with KRU Hidromorava and the Contractor, shall disclose relevant information on the Subproject scope, schedule, expected duration of works, access restrictions, potential water supply interruptions, community health and safety measures and grievance channels. Disclosure shall be undertaken through appropriate channels, including the FLOWS Project website, KRU Hidromorava communication channels, municipal/community notice mechanisms and site notices where relevant. Information shall be made available in languages understandable to affected stakeholders. Stakeholder engagement shall be proportionate to the limited scope and temporary nature of the priority rehabilitation work. Stakeholder engagement activities and a Grievance Mechanism will be maintained throughout Subproject implementation. |

*Although ESS8 is not considered applicable, a Chance Finds Procedure shall be implemented as a precautionary measure.

## 3. SUBPROJECT ENVIRONMENTAL AND SOCIAL BASELINE CONDITIONS

### 3.1 Subproject Setting

Perlepnica Dam is located on the Perlepnica River, approximately 7 km northeast of Gjilan and is operated by the Regional Water Company Hidromorava. Constructed between 1979 and 1982, the dam forms the Perlepnica Reservoir, which serves as an important source of drinking water for communities supplied through the KRU Hidromorava service area.

The proposed rehabilitation works will be undertaken entirely within the existing operational footprint of the dam and associated infrastructure. The Subproject focuses on rehabilitation of priority dam safety components identified during the dam safety assessment, including upgrading of the seepage monitoring system, repair of deteriorated spillway structures, rehabilitation of the terminal section of the spillway chute and plunge pool, repair of leakage points within the bottom outlet gallery, replacement of deteriorated hydromechanical equipment and completion of monitoring instrumentation and associated monitoring systems.

No modifications to the embankment dam body, reservoir storage capacity, spillway capacity or overall dam configuration are envisaged under the Project.

Fig.1 - Perlepnica dam and auxiliary works

Source: Studio Pietrangeli, Dam Safety Assessment and Detailed Design for Perlepnica Dam (2026).

### 3.2 Physical Environment

Topography and Geology

The subproject area is characterized by moderately hilly terrain and narrow valleys typical of eastern Kosovo. Perlepnica Dam is a zoned rock-fill embankment dam with a central clay core, a maximum height of approximately 40 m and a crest length of 165 m. The dam and associated facilities are situated within an established operational area that has been previously disturbed through dam construction, operation and maintenance activities.

Rehabilitation works will be limited to existing operational structures, including the spillway, intake gallery, hydraulic infrastructure and monitoring facilities. No significant earthworks, excavation of new areas or modifications to the dam embankment are envisaged under the Project.

Climate

The area is characterized by a continental climate with warm summers and cold winters. Seasonal precipitation and temperature variations may influence the scheduling and implementation of rehabilitation activities, particularly concrete repair works, crack injection works, rehabilitation of spillway structures and replacement of hydraulic components.

Construction activities may require temporary adjustment during periods of heavy rainfall, freezing temperatures or adverse weather conditions to ensure quality control and worker safety.

Air Quality

The subprojectarea is located outside major industrial and urban pollution sources and generally experiences good ambient air quality conditions.

Temporary and localized impacts on air quality may occur during rehabilitation activities involving surface preparation, concrete removal, drilling, cutting, transportation of materials and operation of construction machinery. These impacts are expected to be short-term and limited to the construction period.

Noise Environment

Existing noise levels within the subproject area are generally low and are associated primarily with dam operation, maintenance activities and occasional local traffic.

Temporary increases in noise levels are expected during rehabilitation works, particularly during drilling, concrete repair works, equipment installation and replacement of hydraulic components. These impacts are expected to be localized, temporary and reversible.

### 3.3 Water Resources

Perlepnica Reservoir is the most sensitive environmental receptor associated with the Project, as it serves as an important source of drinking water within the KRU Hidromorava service area.

The reservoir has a catchment area of approximately 62 km² and a storage capacity of about 4.2 million m³. The reservoir is operated primarily for water supply purposes and covers an area of approximately 21.24 ha at the normal operating level. The normal reservoir level is approximately 596 m above sea level, while the average river runoff at the dam site is approximately 0.55 m³/s.

The dam is equipped with a side-channel concrete spillway with a sill level at elevation 596 m above sea level, a lateral spillway length of approximately 45 m, a spillway channel length of approximately 190 m and a design discharge capacity of approximately 274 m³/s.

The rehabilitation works will be undertaken within existing operational facilities and will not modify reservoir capacity or operating conditions. Particular attention will be required during crack injection works, spillway rehabilitation, replacement of hydraulic components and monitoring system upgrades to prevent accidental release of pollutants, construction materials or wastes into reservoir waters.

The Subproject is expected to provide long-term benefits through improved seepage monitoring capability, rehabilitation of deteriorated infrastructure and enhanced operational reliability of the dam and associated water supply facilities.

### 3.4 Biological Environment

The proposed rehabilitation works will be confined to existing operational dam infrastructure and previously disturbed areas. No impacts on protected areas, natural habitats or sensitive biodiversity receptors are anticipated.

Significant impacts on flora, fauna or aquatic ecosystems are therefore not expected.

3.5 Socio-Economic Environment

Perlepnica Dam is an important component of the regional water supply system and provides essential drinking water services to communities within the KRU Hidromorava service area.

The rehabilitation works are expected to generate limited temporary employment during construction and provide long-term benefits through improved dam safety, enhanced monitoring capability and increased reliability of water supply services.

No permanent land acquisition, physical displacement economic displacement or livelihood impacts are anticipated. Any temporary access restrictions shall be limited to safety exclusion zones within the existing operational footprint and shall be communicated in advance to affected personnel or users, where applicable.

3.6 Existing Dam Safety Conditions

Dam safety assessments and technical investigations undertaken under the FLOWS Project identified a number of deficiencies affecting selected structural, hydraulic and monitoring components of Perlepnica Dam. Identified issues included seepage manifestations at the right abutment, deterioration and cracking of spillway structures, erosion of the terminal section of the spillway chute and plunge pool, seepage within the bottom outlet gallery, corrosion of hydraulic components and deficiencies in the monitoring system.

Based on these findings, priority rehabilitation measures were selected to improve monitoring capability, rehabilitate deteriorated infrastructure and support the long-term safety and operational reliability of the dam and associated water supply infrastructure.

## 4. ENVIRONMENTAL AND SOCIAL RISKS AND IMPACTS

### 4.1 Overview

The rehabilitation works may result in a range of temporary environmental and social impacts associated with construction activities, use of materials and equipment, and the operation of an active dam facility. The principal environmental risks relate to waste generation, dust and noise emissions, protection of reservoir water quality, and the handling of construction materials and hazardous substances. The main social risks are associated with occupational health and safety, worker management and community health and safety.

The identified risks are expected to be localized, short-term and manageable through the implementation of good construction practices and the mitigation measures defined in this ESMP. The Subproject is also expected to provide long-term benefits through improved dam safety, enhanced seepage monitoring capability, rehabilitation of deteriorated infrastructure and improved operational reliability.

### 4.2 Environmental Risks and Impacts

4.2.1 Dust Emissions

Temporary dust emissions may be generated during concrete repair works, surface preparation, drilling, grinding, crack repair activities, rehabilitation of the spillway structures and plunge pool, and transportation of materials.

Dust generation may affect workers undertaking rehabilitation activities and, to a lesser extent, nearby operational areas within the dam facility. Due to the limited scale and localized nature of the works, impacts on ambient air quality are expected to be temporary and confined to the immediate work areas.

No significant long-term impacts on air quality are anticipated.

4.2.2 Noise and Vibration

Construction activities involving drilling, grinding, concrete repair works, replacement of hydraulic components, installation of monitoring equipment and operation of construction machinery may generate temporary increases in noise levels.

Potential receptors include subproject workers, operational personnel and visitors within the dam complex. Given the absence of dense residential development within the immediate subproject area and the limited duration of rehabilitation activities, noise impacts are expected to be temporary and localized.

Significant vibration impacts are not anticipated due to the nature and scale of the proposed works.

4.2.3 Water Quality Risks

Perlepnica Reservoir is the most sensitive environmental receptor associated with the Project, as it serves as an important source of potable water supply for communities served by KRU Hidromorava.

Construction activities undertaken within or near hydraulic structures may create risks of accidental release of construction waste, concrete residues, sediments, oils, fuels, grouts, coatings or other materials into the reservoir. Improper management of such materials could adversely affect water quality and water supply infrastructure.

Although the proposed activities do not involve modifications to reservoir capacity, water abstraction rates or operational regimes, protection of reservoir water quality remains a key environmental consideration throughout Subproject implementation

4.2.4 Construction Waste Generation

Implementation of the proposed rehabilitation works will generate various types of construction waste. Waste streams are expected to include concrete debris from repair works on spillway structures and the bottom outlet gallery, scrap metal from replacement of pipes, valves and associated hydraulic components, packaging materials associated with equipment installation, and limited quantities of waste generated during monitoring system rehabilitation works.

Improper handling, storage or disposal of construction waste could result in localized environmental impacts, including visual disturbance, soil contamination and potential contamination of reservoir waters. Given that the rehabilitation works will be undertaken within an operational drinking water facility, effective management of construction waste is particularly important.

The anticipated impacts are expected to be temporary, localized and reversible.

4.2.5 Hazardous Materials and Associated Waste

Several rehabilitation activities will involve the use of chemical products and potentially hazardous materials, including grouts, sealants, coatings, paints, solvents, lubricants and cleaning agents. Associated activities may also generate limited quantities of hazardous and electronic waste, including contaminated packaging, paint residues, used solvents, empty containers, cables and obsolete monitoring or hydraulic equipment.

Risks may arise during the transportation, storage, handling, application and disposal of these materials and wastes. Improper management could result in contamination of soil and water resources, adverse effects on worker health and safety, and localized environmental impacts.

Given the location of the works within an operational drinking water facility, appropriate management of hazardous materials and associated waste streams will be required throughout Subproject implementation. The anticipated impacts are expected to be temporary, localized and manageable through implementation of appropriate environmental management measures.

### 4.3 Social Risks and Impacts

4.3.1 Occupational Health and Safety Risks

The proposed rehabilitation activities will be undertaken within an operational dam facility and involve civil, mechanical and monitoring works. Subproject workers may be exposed to a range of occupational hazards, including:

- Working at height;

- Confined-space entry;

- Electrical hazards;

- Manual handling and lifting operations;

- Exposure to grouts, coatings, solvents and other chemical products;

- Slips, trips and falls within operational areas.

Failure to implement appropriate occupational health and safety procedures could result in injuries, accidents or adverse health impacts affecting workers and contractors. Effective implementation of OHS measures will therefore be essential throughout Subproject implementation.

4.3.2 Community Health and Safety Risks

Potential community-related risks are expected to be limited and mainly associated with temporary disruption of water supply services during priority rehabilitation works, localized water quality impacts in case of accidental spills or improper handling of materials near reservoir or water supply infrastructure, movement of vehicles and equipment on access roads, unauthorized access to active work areas, and possible miscommunication regarding planned operational restrictions. Given the limited scope and temporary nature of the priority interventions, significant community health and safety impacts are not anticipated. These risks shall be managed through close coordination with KRU Hidromorava, advance planning and notification of operational activities, implementation of water quality protection and spill-prevention measures, traffic and access control, signage, fencing, and clear public communication where service interruptions or restrictions are required.

Where planned water supply interruptions are unavoidable, KRU Hidromorava shall notify affected consumers in advance, indicate the expected timing and duration of the interruption where practicable, and identify any critical or vulnerable users requiring special consideration. In case of accidental water quality impact or unplanned service disruption, KRU Hidromorava, the PMT, the Supervising Engineer and the Contractor shall coordinate immediate response, public communication and corrective measures.

4.3.3 Labour and Working Conditions

The Subproject will involve direct workers, contracted workers and supervisory personnel engaged in priority rehabilitation activities. Labour-related risks may include non-compliance with labour requirements, inadequate worker management arrangements, inadequate working hours and rest arrangements, non-payment or delayed payment of wages, insufficient training and awareness, lack of clear employment terms, inadequate occupational health and safety measures, discrimination or unequal treatment, inappropriate worker behaviour, sexual harassment or other misconduct, and the absence of effective worker grievance procedures.

Given the limited scale and temporary nature of the Project, labour and working condition risks are expected to be manageable through compliance with national labour legislation, implementation of appropriate worker management procedures and application of the requirements set out in ESS2, provision of site-specific occupational health and safety measures, use of personal protective equipment, induction and awareness training, supervision of contractors, use of a worker Code of Conduct, prohibition of child labour and forced labour, and establishment of an accessible worker grievance mechanism.

4.4 Dam Safety Related and Operational Risks

Although the proposed interventions are intended to improve dam safety and operational reliability, implementation of rehabilitation works within an active dam and water supply facility may create temporary dam safety and operational interface risks. These may include temporary restrictions on access to operational areas, interference with existing monitoring instruments, temporary unavailability of selected hydraulic, electrical or monitoring components, damage to existing infrastructure, delayed commissioning of new equipment, incorrect sequencing of works, or discovery of previously unidentified defects.

Such risks may have implications for operational continuity, water supply reliability and, in exceptional circumstances, downstream community safety. These risks shall be managed through close coordination with KRU Hidromorava, approval of method statements and work permits, maintenance of critical dam functions, protection of existing monitoring systems, emergency preparedness arrangements, immediate reporting of newly identified defects, and testing, commissioning and handover procedures prior to acceptance of works.

## 5. ENVIRONMENTAL AND SOCIAL MITIGATION MEASURES

### 5.1 General

The mitigation measures presented in this chapter have been developed to avoid, minimize, reduce or manage the environmental and social risks and impacts identified in Chapter 4. The measures have been designed in accordance with the mitigation hierarchy and are consistent with applicable national legislation and the World Bank Environmental and Social Standards (ESSs).

The Contractor shall be responsible for the day-to-day implementation of all mitigation measures during Subproject implementation. The Supervising Engineer shall monitor the Contractor’s compliance on site, while the Project Management Team (PMT), in coordination with KRU Hidromorava, shall provide overall oversight, review monitoring reports, and ensure that the ESMP requirements are effectively implemented.

The mitigation measures focus on the principal environmental and social risks associated with the rehabilitation works, including construction waste management, dust and noise control, protection of water resources, management of hazardous materials, occupational health and safety, labour and working conditions, community health and safety, and safe implementation of rehabilitation works within an operational dam facility.

Table 5-1. Environmental and Social Mitigation Measures

| Source of Impact | Mitigation Measures | Responsibility | Estimated Cost |
| --- | --- | --- | --- |
| Air Quality and Dust Emissions – Dust generated during concrete repair works, drilling, grinding, surface preparation, rust removal and transportation activities. | - Water spraying shall be applied during dry and windy conditions. - Dust-generating materials shall be covered during transportation and storage. - Work areas shall be regularly cleaned. - Vehicle speed limits shall be established and enforced within the work area. - Workers shall use respiratory protective equipment where required. | Contractor | Included in Contractor's BOQ |
| Exhaust Emissions – Emissions from construction machinery, generators and transport vehicles. | - Construction equipment shall be regularly maintained and operated according to manufacturer specifications. - Unnecessary engine idling shall be avoided. - Equipment with excessive emissions shall be removed from service until repaired. | Contractor | Included in Contractor's BOQ |
| Noise and Vibration – Noise generated by drilling, grinding, concrete rehabilitation, valve replacement, electrical installation and operation of construction machinery. | - Noisy activities shall be restricted to daytime working hours where feasible. - Equipment shall be properly maintained and fitted with silencers where appropriate. - Workers exposed to elevated noise levels shall be provided with hearing protection. - Activities involving drilling, grinding or mechanical removal of deteriorated concrete shall be undertaken in a controlled manner to avoid damage to adjacent structures and equipment. | Contractor | Included in Contractor's BOQ |
| Water Quality – Accidental release of concrete debris, slurry, grout, repair materials, sediments, fuels, oils, lubricants, paints, coatings or chemicals into Perlepnica Reservoir. | - Physical containment measures shall be implemented where necessary. - Fuel and chemical storage areas shall be located away from water bodies and equipped with secondary containment. - Spill kits shall be available at all work locations. - Any spill shall be immediately reported, contained and cleaned. Direct disposal of waste into the reservoir shall be prohibited. - No refueling, equipment maintenance or washing activities shall be undertaken within the immediate vicinity of Perlepnica Reservoir. | Contractor | Included in Contractor's BOQ |
| Sewage and Wastewater – Wastewater generated by construction personnel and cleaning activities. | - Existing sanitary facilities shall be used where available. - Mobile sanitary facilities shall be provided where existing facilities are not accessible. - Direct discharge of wastewater into Perlepnica Reservoir shall be strictly prohibited. - Wastewater and cleaning residues shall be collected and managed in accordance with applicable regulations. - A zero-discharge policy to the reservoir shall be applied. | Contractor | Included in Contractor's BOQ |
| Solid Waste – Waste generated from spillway rehabilitation works, repair works within the bottom outlet gallery, removed pipes, valves, metallic components, monitoring equipment and packaging materials. | - A Waste Management Plan shall be implemented. - Construction waste shall be segregated at source. - Scrap metal, removed valves and pipes shall be collected separately and transferred for recycling where feasible. - Waste shall be stored in designated areas and disposed of through licensed waste operators. - Waste tracking records shall be maintained. | Contractor | Included in Contractor's BOQ |
| Hazardous Materials Management – Storage and use of grouts, sealants, anti-corrosion coatings, paints, solvents, lubricants, cleaning agents and other hazardous substances. | - Safety Data Sheets (SDS) shall be available on site. - Hazardous materials shall be stored in designated and secure areas with secondary containment. - Only trained personnel shall handle hazardous substances. - Hazardous materials shall be managed in accordance with manufacturer recommendations. | Contractor | Included in Contractor's BOQ |
| Hazardous Waste – Generation of contaminated containers, paint residues, used solvents, absorbents and chemical packaging. | - Hazardous waste shall be separately collected, labelled and stored in secure locations. - Hazardous waste shall be transferred only to licensed waste management operators. - Mixing of hazardous and non-hazardous waste shall be prohibited. | Contractor | Included in Contractor's BOQ |
| Electronic Waste – Replacement or upgrading of monitoring instruments, sensors, cables and associated monitoring equipment. | - Electronic waste shall be separately collected and transferred to authorized recycling or disposal facilities. - Records of electronic waste management and disposal shall be maintained. | Contractor | Included in Contractor's BOQ |
| Corrosion Protection Works – Hydromechanical Rehabilitation and Corrosion Protection Works – Removal and replacement of pipes, valves and associated hydraulic components, including surface preparation, cleaning and application of protective coatings where required. | - Surface preparation residues shall be collected and disposed of appropriately. - Application of coatings and bituminous materials shall be undertaken in accordance with manufacturer specifications. - Measures shall be implemented to prevent release of residues or chemicals into the reservoir. | Contractor | Included in Contractor's BOQ |
| Temporary Storage of Materials and Equipment – Storage of pipes, valves, monitoring equipment, electrical components and rehabilitation materials within the dam operational area. | - Materials shall be stored only in designated areas approved by the Supervising Engineer and coordinated with KRU Hidromorava. - Storage shall not obstruct access to operational facilities, monitoring systems, inspection routes or emergency access routes. - Good housekeeping practices shall be maintained at all times. | Contractor | Included in Contractor's BOQ |
| Dam Infrastructure and Operational Environment – Damage to existing hydraulic structures, monitoring systems or operational facilities during rehabilitation works. | - All works shall be carried out in accordance with approved technical designs, method statements and supervision requirements. - Existing monitoring systems shall remain operational wherever practicable. - Any unexpected damage shall be immediately reported to the Supervising Engineer and KRU Hidromorava. | Contractor / Supervising Engineer | Included in Contractor's BOQ |
| Monitoring Instrumentation Installation – Potential malfunction or damage to monitoring systems during installation and commissioning activities. | - Installation and commissioning shall be undertaken by qualified personnel. - Functional testing, calibration and verification of monitoring instruments shall be completed before acceptance of the works and integrated into the data acquisition system where applicable. | Contractor | Included in Contractor's BOQ |
| Traffic and Transport – Delivery of valves, pipes, monitoring instruments, electrical equipment, coatings and construction materials to the dam site. | - Vehicle movements shall be planned and managed to minimize safety and environmental risks. - Loads shall be properly secured during transportation. - Drivers shall comply with applicable traffic regulations and site speed limits. | Contractor | Included in Contractor's BOQ |
| Community Health and Safety – Risks associated with rehabilitation works within an operational drinking water facility, including unauthorized access to work areas, accidental spills, temporary operational disruptions and potential impacts on water supply services. | - Access to active work areas shall be restricted through fencing, barriers and warning signage. - Construction activities shall be coordinated with KRU Hidromorava to ensure continued safe operation of the facility and minimize disruption to water supply services. - Emergency procedures shall be established to address accidental spills, incidents or operational disruptions. - Planned water supply interruptions shall be avoided where feasible and minimized in duration. - Where planned interruptions are unavoidable, affected consumers shall be informed in advance through KRU Hidromorava established communication channels, municipal/community channels and other appropriate public notification mechanisms. Notifications shall include the expected timing, duration and reason for interruption, where practicable. - The expected timing and duration of the interruption shall be communicated whenever practicable. - Unauthorized persons shall be prohibited from entering work areas. - The Contractor shall immediately notify the Supervising Engineer, KRU Hidromorava and PMT of any incident that may affect public safety, water quality or continuity of water supply services. - A Grievance Mechanism (GM) shall be available to affected stakeholders. - Vulnerable consumers shall be considered in planning any prolonged interruption or emergency response. - Public communication shall be clear and factual and shall avoid creating unnecessary concern regarding dam safety. - In case of accidental water quality impact, spill or unplanned service disruption, the Contractor shall immediately notify the Supervising Engineer, KRU Hidromorava and PMT, and emergency communication procedures shall be activated. | Contractor / KRU Hidromorava | Included in Contractor's BOQ |
| Occupational Health and Safety (OHS) – Work at height, confined space entry, lifting operations, electrical works, concrete rehabilitation works and exposure to hazardous materials. | - A site-specific OHS Plan shall be prepared and implemented. - Appropriate PPE shall be provided to all workers. - Work-at-height procedures, confined-space entry permits, lock-out/tag-out procedures and lifting plans shall be implemented where applicable. - Electrical works shall be undertaken only by qualified personnel. - First aid equipment and trained first aid personnel shall be available on site. | Contractor | Included in Contractor's BOQ |
| Worker Health Concerns – Exposure to dust, noise, chemicals, coatings, solvents and physically demanding working conditions. | - Workers shall receive task-specific safety training and information on workplace hazards. - Appropriate PPE shall be used and maintained. - Safety Data Sheets (SDS) shall be available for all hazardous substances used on site. - Adequate drinking water, sanitation facilities and rest areas shall be provided. | Contractor | Included in Contractor's BOQ |
| Worker Code of Conduct and Labour Management – Inappropriate worker behaviour, discrimination, harassment, unsafe practices or non-compliance with labour requirements. | - The Code of Conduct shall include provisions on respectful behaviour, non-discrimination, prohibition of harassment, prohibition of sexual exploitation and abuse/sexual harassment, prohibition of violence, safe interaction with communities and compliance with site safety rules. - All workers shall receive site induction training prior to commencement of work. - A worker Code of Conduct shall be implemented and signed by all personnel. - Contractors shall comply with applicable labour legislation and ensure equal opportunity, non-discrimination and fair treatment of workers. - Breaches of the Code of Conduct shall be subject to disciplinary measures in accordance with labour legislation and contract requirements. | Contractor | Included in Contractor's BOQ |
| Worker Grievance Mechanism – Failure to address worker concerns, complaints or labour-related issues. | - A worker grievance mechanism shall be established and communicated to all subproject workers. - Complaints shall be recorded, investigated and resolved in a timely manner. - Records of grievances and corrective actions shall be maintained. - The WGM shall include indicative response timelines and escalation arrangements. - Grievances involving harassment, SEA/SH or other sensitive matters shall be handled confidentially and only by personnel designated to manage such cases. - Confidential and sensitive grievances shall be handled through restricted-access procedures. - Retaliation against complainants shall be prohibited. | Contractor | Included in Contractor's BOQ |
| Security and Access Control – Temporary restrictions on access to operational facilities, inspection routes and work areas during rehabilitation activities. Use of barriers, gates, guards or access restrictions around operational and construction areas. | - Rehabilitation works shall be planned and phased to minimize disruption to dam operations and maintenance activities. - Safe access shall be maintained for authorized personnel, emergency response teams and dam operators at all times. - Temporary restrictions shall be communicated in advance to affected personnel. - Access control measures shall be proportionate to the risks and limited to safety and operational requirements. - Security personnel, where used, shall be instructed on respectful conduct, non-discrimination, avoidance of excessive force and referral of complaints to the Subproject GM. - Access restrictions shall not obstruct emergency access, dam operation or authorized inspection routes. - Signage shall clearly indicate restricted areas and alternative safe routes where relevant. | Contractor / KRU Hidromorava / Supervising Engineer | Included in Contractor's BOQ |
| Emergency Preparedness and Response – Workplace accidents, fire, chemical spills, electrical incidents, confined-space emergencies or other unforeseen events. | - The Contractor shall prepare and implement an Emergency Preparedness and Response Plan. - Emergency contact information, evacuation procedures, rescue equipment and emergency response arrangements shall be available on site. - Emergency drills shall be conducted periodically throughout implementation of the works. | Contractor | Included in Contractor's BOQ |
| DAM OPERATION – Temporary disruption of dam operation, water supply infrastructure or hydraulic systems during rehabilitation activities. | - Rehabilitation activities shall be planned and coordinated with KRU Hidromorava to ensure continued safe operation of the dam and water supply system. - Any temporary shutdowns or operational restrictions shall be approved in advance and implemented in accordance with agreed operational procedures. - Critical dam functions shall remain operational throughout the construction period wherever practicable. | Contractor / KRU Hidromorava / Supervising Engineer | Included in Contractor's BOQ |
| Monitoring Systems – Temporary interruption, damage or malfunction of existing dam monitoring systems during installation of new instrumentation. | - Existing monitoring instruments shall remain operational wherever practicable during installation works. - Installation, calibration and commissioning shall be carried out by qualified personnel. - Functionality of all instruments shall be verified following installation and prior to acceptance of the works. - Monitoring data shall be reviewed to confirm proper operation of the upgraded system. | Contractor / Supervising Engineer / KRU Hidromorava | Included in Contractor's BOQ |
| Newly Identified Defects – Discovery of previously unidentified structural, hydraulic, geotechnical or operational deficiencies during rehabilitation works. | - Any newly identified defects or safety concerns shall be immediately reported to the Supervising Engineer and KRU Hidromorava. - Appropriate technical assessments shall be undertaken to determine the significance of the defect and identify corrective measures. - Rehabilitation activities in the affected area shall be suspended where necessary until the issue has been evaluated and appropriate actions agreed. | Contractor / Supervising Engineer / KRU Hidromorava | Included in Contractor's BOQ |

6. ENVIRONMENTAL AND SOCIAL MONITORING PLAN

6.1 Monitoring Objectives

The objective of the monitoring program is to verify the effective implementation of the mitigation measures defined in Chapter 5 and to ensure that environmental, social, occupational health and safety, and dam safety risks associated with the rehabilitation works are adequately managed throughout subproject implementation.

Monitoring activities shall be undertaken by the Contractor, supervised by the Supervising Engineer and reviewed by KRU Hidromorava and the PMT, as applicable. Monitoring results shall be documented and maintained as part of the Project’s environmental and social management records and shall be made available to the World Bank upon request.

Table 6.1 Environmental and Social Monitoring Plan

| Parameter to be Monitored | Monitoring Method | Location | Frequency | Responsibility |
| --- | --- | --- | --- | --- |
| Dust generation and effectiveness of dust suppression measures | Visual inspection of work areas and dust control practices | Active work areas | Weekly and after complaints | Contractor |
| Vehicle and equipment emissions | Visual inspection and review of maintenance records | Construction site | Monthly | Contractor |
| Noise levels and implementation of noise control measures | Site inspection and spot measurements where required | Active work areas | Weekly | Contractor |
| Waste segregation and temporary storage | Inspection of waste storage areas and waste handling practices | Construction site | Weekly | Contractor |
| Waste disposal records | Review of waste transfer notes and disposal documentation | Construction site | Monthly | Contractor / Supervising Engineer |
| Hazardous waste management | Inspection of hazardous waste storage and disposal records | Construction site | Weekly | Contractor |
| Electronic waste management | Review of collection, storage and disposal records | Construction site | Monthly | Contractor/ Supervising Engineer |
| Water quality protection measures | Inspection of containment measures, housekeeping practices and reservoir protection arrangements | Reservoir and work areas | Weekly | Contractor / Supervising Engineer / KRU Hidromorava |
| Raw water quality | Review of routine raw water quality monitoring records and investigation of any changes potentially linked to construction activities | Reservoir intake area | Weekly during construction and as required | KRU Hidromorava/ Supervising Engineer |
| Evidence of spills, leaks or accidental releases | Inspection of fuel storage, chemical storage and work areas | Construction site | Weekly | Contractor |
| Hazardous materials storage and handling | Inspection of storage facilities, secondary containment and SDS availability | Construction site | Weekly | Contractor |
| Corrosion protection and coating works | Inspection of material handling, residue collection and housekeeping practices | Pipe rehabilitation areas | Weekly | Contractor |
| Temporary storage of construction materials and equipment | Inspection of designated storage areas and housekeeping practices | Construction site | Weekly | Contractor |
| Protection of existing dam infrastructure during rehabilitation works | Inspection of work practices and condition of adjacent structures | Dam facilities | Weekly | Supervising Engineer / KRU Hidromorava |
| Monitoring instrument installation and commissioning | Review of installation records, calibration reports and functionality tests | Dam monitoring facilities | During installation and commissioning | Contractor / Supervising Engineer / KRU Hidromorava |
| Seepage monitoring system functionality | Review of monitoring records, instrument readings, calibration reports and operational status of seepage monitoring facilities | Dam monitoring facilities | Monthly and during commissioning | Contractor / Supervising Engineer / KRU Hidromorava |
| Environmental and social incidents. Serious incidents and accidents, including OHS, CHS, water quality, service disruption and SEA/SH-related incidents. | Review of incident records and corrective actions. Incident notification records, investigation reports, root-cause analysis and corrective action tracking. | Construction site | Immediately following occurrence and as required | Contractor / Supervising Engineer/ KRU Hidromorava / PMT |
| Social |  |  |  |  |
| Use of personal protective equipment (PPE) | Site inspections | Work areas | Daily | Contractor |
| Compliance with OHS requirements | Safety inspections and toolbox meetings | Construction site | Daily | Contractor |
| Work-at-height permits and confined-space permits | Review of permit records | Construction site | Daily (as required) | Contractor |
| Worker induction and training | Review of training records | Construction site | Monthly | Contractor |
| Worker grievance mechanism | Review of grievance register | Construction site | Monthly | Contractor |
| Worker Code of Conduct implementation and SEA/SH awareness | Review of signed records and awareness activities | Construction site | Monthly | Contractor |
| Community safety measures (signage, barriers and access control) | Site inspection | Work areas | Daily (as required) | Contractor |
| Community grievances | Review of grievance register and resolution records | Subprojectarea | Monthly | PMT (with support from KRU Hidromorava where relevant) |
| Access management and restrictions | Inspection of access arrangements and operational access routes | Dam facilities | Daily (as required) | Contractor / KRU Hidromorava |
| Emergency preparedness arrangements | Inspection of emergency equipment and response arrangements | Construction site | Monthly | Contractor |
| Occupational accidents and incidents | Review of incident records and corrective actions | Construction site | Monthly and immediately following any reportable incident | Contractor / PMT |
| Public notification of planned water supply interruptions | Review of public notices, KRU communication records and consumer notifications. | Subprojectarea | Before and during any planned interruption. | Contractor / KRU Hidromorava / PMT |
| Dam Safety and Operational Monitoring |  |  |  |  |
| Continuity of dam operations during rehabilitation works | Coordination meetings and site inspections | Dam facilities | Weekly | KRU Hidromorava / Supervising Engineer |
| Functionality of existing dam monitoring systems | Review of monitoring records and operational status | Dam monitoring facilities | Weekly | KRU Hidromorava |
| Installation and performance of new monitoring instruments | Verification of calibration, testing, commissioning and data acquisition system integration records | Dam monitoring facilities | During installation and commissioning | Contractor / Supervising Engineer / KRU Hidromorava |
| Temporary operational restrictions and shutdowns | Review of work permits and operational records and records of consumer notifications, where applicable | Dam facilities | As required | Contractor / KRU Hidromorava |
| Communication of planned water supply interruptions (where applicable). | Review of public notices, customer notifications and operational records | Subprojectarea | As required | KRU Hidromorava / Contractor |
| Newly identified defects or safety concerns | Site inspections and technical assessment reports | Dam facilities | Continuous during works | Contractor / Supervising Engineer / KRU Hidromorava |
| Emergency preparedness and response capability | Review of emergency procedures, equipment and drill records | Dam facilities | Quarterly | Contractor / KRU Hidromorava |
| Corrective actions arising from inspections or incidents | Review of inspection reports and corrective action records | Dam facilities | Monthly | PMT / Supervising Engineer |
| ESMP implementation progress and compliance status | Review of monitoring reports, inspections and corrective actions | Subproject level | Monthly | PMT |
| Environmental and Social reporting to the World Bank | Review of monitoring records and subproject reports | Subproject level | Quarterly and as required | PMT |

7. INSTITUTIONAL ARRANGEMENTS, CAPACITY BUILDING AND TRAINING

## 7.1 Institutional Arrangements

The implementation of this ESMP shall involve several institutions and entities with responsibilities for environmental, social, occupational health and safety (OHS), and dam safety management throughout subproject implementation.

The Ministry of Environment and Spatial Planning (MESP), through the Project Management Team (PMT), shall be responsible for overall subproject coordination and oversight, including supervision of ESMP implementation and reporting to the World Bank.

The Contractor shall be responsible for the day-to-day implementation of all environmental, social, health and safety (ESHS) measures defined in this ESMP and incorporated into the construction contract. The Contractor shall ensure compliance with applicable environmental, social, occupational health and safety requirements and shall maintain adequate records demonstrating implementation of ESMP measures.

The Supervising Engineer shall be responsible for monitoring compliance with the ESMP, conducting site inspections, verifying implementation of mitigation measures, reviewing monitoring reports and recommending corrective actions where necessary.

Regional Water Company Hidromorava, as the operator and beneficiary of Perlepnica Dam, shall coordinate access to operational facilities, monitor potential impacts on dam operations and water supply services, and participate in the review, testing and acceptance of rehabilitation works, including monitoring instrumentation and dam safety systems.

The environmental and social responsibilities of the key institutions involved in Subproject implementation are summarized in Section 7.2 below.

## 7.2 Institutional Responsibilities

The successful implementation of the ESMP requires clear allocation of responsibilities among the institutions involved in Subproject implementation, supervision and operation. The principal responsibilities of each institution are summarized in Table 7-1.

Table 7-1. Institutional Responsibilities

| Institution | Key Responsibilities |
| --- | --- |
| MESP / PMT | Overall Subproject coordination and oversight; supervision of ESMP implementation; review of environmental and social monitoring reports; management of grievances; coordination with the World Bank; monitoring implementation of corrective actions; ensuring compliance with ESF requirements. |
| Contractor | Implementation of all mitigation and monitoring measures defined in the ESMP; preparation and implementation of the Contractor's Environmental and Social Management Plan (C-ESMP); preparation of site-specific ESHS procedures and plans; waste management; occupational health and safety management; worker training; incident reporting; maintenance of environmental and social records; implementation of corrective actions. |
| Supervising Engineer | Monitoring and verification of ESMP implementation; regular site inspections; review of Contractor's reports; verification of compliance with environmental, social, health and safety requirements; identification of non-compliance and recommendation of corrective actions; supervision of rehabilitation works. |
| KRU Hidromorava | Coordination of access to operational facilities; monitoring impacts on dam operation and water supply services; participation in testing, commissioning and operational acceptance of rehabilitation works, including monitoring instrumentation and dam safety-related systems; coordination of operational and emergency response arrangements during implementation; coordination of communication with consumers in relation to planned service interruptions, where applicable. |

### 7.3 Capacity Building and Training

The environmental and social risks associated with the proposed rehabilitation works are considered manageable and largely related to construction activities within an existing operational dam facility. Nevertheless, adequate training and awareness activities are necessary to ensure effective implementation of the ESMP and compliance with applicable environmental, social, occupational health and safety, and dam safety requirements.

The Contractor shall be responsible for providing all required training to Subproject personnel prior to and during implementation of the rehabilitation works. The Supervising Engineer shall verify that required training has been delivered and appropriately documented. Records of training activities, including attendance sheets, training materials and training dates, shall be maintained on site and made available for review by the Supervising Engineer and PMT.

Table 7-2. Training Requirements

| Training Topic | Target Group | Training Provider | Timing |
| --- | --- | --- | --- |
| ESMP requirements and environmental compliance | Contractor personnel and site management | Contractor | Prior to commencement of works |
| Occupational Health and Safety (OHS) | All workers | Contractor | Prior to commencement of works and periodically during implementation |
| Working at heights procedures | Relevant workers | Contractor | Before commencement of relevant activities |
| Confined space entry procedures | Relevant workers | Contractor | Before commencement of relevant activities |
| Hazardous materials handling and spill response | Relevant workers | Contractor | Prior to use of hazardous substances |
| Emergency preparedness and response | Contractor personnel and site management | Contractor | Prior to commencement of works and periodic refresher training |
| Code of Conduct and worker behaviour | All workers | Contractor | During induction training |
| Dam safety awareness and emergency preparedness | Contractor personnel, Supervising Engineer and relevant KRU staff | Contractor and KRU Hidromorava | Prior to commencement of rehabilitation activities |

The Contractor shall maintain records of all training activities, including attendance sheets, training materials and training dates. These records shall be made available to the Supervising Engineer, PMT and the World Bank upon request.

### 7.4 Reporting Arrangements

The Contractor shall maintain records and report on implementation of the ESMP, including environmental, social, occupational health and safety, and dam safety matters. The Supervising Engineer shall monitor compliance and verify implementation of mitigation measures and corrective actions.

The PMT shall oversee ESMP implementation, review monitoring information and ensure reporting to the World Bank in accordance with Subproject requirements. Significant incidents and accidents shall be reported promptly in accordance with applicable reporting procedures.

The Contractor shall immediately notify the Supervising Engineer, KRU Hidromorava and PMT of any serious incident or accident, including fatality, serious injury, major spill, confirmed or suspected water quality impact, unplanned water supply interruption, fire, electrical incident, confined-space incident, security incident, community health and safety incident, or SEA/SH allegation. The PMT shall notify the World Bank in accordance with the Project’s incident reporting requirements and shall ensure preparation of a root-cause analysis and corrective action plan where required.

## 8. GRIEVANCE REDRESS MECHANISM (GRM)

A Grievance Redress Mechanism (GRM) shall be maintained throughout Subproject implementation to enable local communities, stakeholders and other interested parties to submit complaints, concerns, requests for information or suggestions related to Subproject activities.

The GRM established by the Project Management Team (PMT) within MESP shall be used for this Subproject. Information regarding the grievance mechanism shall be disclosed to affected communities and stakeholders prior to commencement of works and shall remain available throughout Subproject implementation.

Grievances may be submitted through the following channels:

- Online grievance form: https://flows-ks.info/forma-e-ankeses/

- Telephone: +383 48 600 883;

- E-mail complaints@flows-ks.info;

- Written submissions to the PMT (Hajdar Dushi, 1-B Nr.7, 10000 Prishtina);

- Direct communication with the PMT during site visits, consultations and stakeholder meetings.

All grievances received shall be recorded in a grievance register and reviewed by the PMT. Grievances related to construction activities shall be coordinated with the Contractor, Supervising Engineer and KRU Hidromorava, as appropriate.

The PMT shall be responsible for maintaining grievance records, monitoring the resolution process and ensuring that grievances are addressed in a timely manner. Information on grievance management and resolution shall be included in periodic Subproject monitoring reports.

The Contractor shall establish and maintain a separate Worker Grievance Mechanism as part of the Contractor's Environmental and Social Management Plan (C-ESMP), in accordance with ESS2 and applicable labour legislation. The WGM shall be communicated to all Subproject workers during induction and shall provide a confidential mechanism for raising workplace concerns and complaints.

## 9. IMPLEMENTATION SCHEDULE AND INTEGRATION OF ESMP INTO SUBPROJECT IMPLEMENTATION

### 9.1 ESMP Implementation Schedule

The mitigation and monitoring measures defined in this ESMP shall be implemented throughout Subproject implementation, including pre-construction, construction, commissioning and operation phases, as applicable.

Prior to commencement of works, the Contractor shall prepare and submit a Contractor's Environmental and Social Management Plan (C-ESMP) consistent with the requirements of this ESMP. The C-ESMP shall include, as applicable, method statements, waste management procedures, occupational health and safety procedures, emergency preparedness and response arrangements, traffic and access management measures, hazardous materials management procedures and other site-specific environmental and social management measures. The C-ESMP shall be reviewed by the Supervising Engineer and approved by the PMT prior to mobilization.

Table 9-1. ESMP Implementation Schedule

| ESMP Activity | Pre-Construction | Construction | Commissioning | Operation |
| --- | --- | --- | --- | --- |
| Incorporation of ESMP requirements into bidding and contract documents | ✓ |  |  |  |
| Preparation and approval of Contractor's ESMP (C-ESMP) | ✓ |  |  |  |
| Environmental and social mitigation measures |  | ✓ | ✓ |  |
| Occupational health and safety measures |  | ✓ | ✓ |  |
| Environmental and social monitoring |  | ✓ | ✓ |  |
| Dam safety and operational monitoring |  | ✓ | ✓ | ✓ |
| Training and awareness activities | ✓ | ✓ |  |  |
| Reporting and corrective actions |  | ✓ | ✓ | ✓ |

### 9.2 Integration of ESMP into Subproject Implementation

The ESMP shall form an integral part of Subproject implementation. All environmental, social, health and safety requirements identified in this ESMP shall be incorporated into the bidding documents, technical specifications and construction contract.

The Contractor shall be responsible for implementing all environmental, social, health and safety measures required during rehabilitation works as part of its contractual obligations, including implementation of the approved C-ESMP.

Compliance with the ESMP shall be monitored throughout implementation by the Contractor, Supervising Engineer, PMT and KRU Hidromorava, as applicable.

Implementation of the ESMP shall be considered a contractual obligation and failure to comply with ESMP requirements may result in corrective actions in accordance with the provisions of the construction contract.